| 27.03 | Подключение светодиода с ограничительным резистором https://www.tinkercad.com/things/63Ts4QJkK3x |  |
| --- | --- | --- |
| 25.03 | Параллельное соединение. Измерения тока и напряжения. |  |
| 30.03 | Микросхемы аналоговые. Компараторы. https://www.tinkercad.com/things/0aqspfRlwKz |  |
| 1.04 | Микросхемы с множеством компараторов. |  |
| 6.04 | Генераторы импульсов. |  |
| 8.04 | Микросхема 555. Мигающий светодиод. |  |
| 13.04 | Выбор элементов RC для создания импульса 1-5 ms. Подключение сервопривода. |  |
| 20.04 | Пропорциональное управление. Модификация предыдущей схемы |  |
| 22.04 | Цифровые схемы. Логика работы. |  |
| 27.04 | Arduino. |  |
| 29.04 | Программирования с использованием языка blockly. https://blocklyduino.github.io/BlocklyDuino/blockly/apps/blocklyduino/ |  |

Программа для проверки сервопривода для Arduino

/*

Sweep

by BARRAGAN <http://barraganstudio.com>

This example code is in the public domain.

modified 8 Nov 2013  by Scott Fitzgerald

http://www.arduino.cc/en/Tutorial/Sweep

*/

#include <Servo.h>

int pos = 0;

Servo servo_9;

void setup()

{

servo_9.attach(9);

}

void loop()

{

// sweep the servo from 0 to 180 degrees in steps

// of 1 degrees

for (pos = 0; pos <= 180; pos += 1) {

// tell servo to go to position in variable 'pos'

servo_9.write(pos);

// wait 15 ms for servo to reach the position

delay(15); // Wait for 15 millisecond(s)

}

for (pos = 180; pos >= 0; pos -= 1) {

// tell servo to go to position in variable 'pos'

servo_9.write(pos);

// wait 15 ms for servo to reach the position

delay(15); // Wait for 15 millisecond(s)

}

}